ENVIRONMENTAL STATUS OF SEDIMENTS IN THE LOWER HAWKESBURY NEPEAN RIVER, NEW SOUTH WALES

C. Matthai1, K. Guise2, P. Coad2, S. McCready1 and S. Taylor3

1URS Australia Pty Ltd, Sydney, NSW; 2Hornsby Shire Council, Sydney, NSW;

3Geochemical Assessments Pty Ltd, Sydney, NSW

Abstract

In common with many other estuarine systems, the Lower Hawkesbury River north of Sydney is under increasing environmental pressure from continued development. Sediments which time-integrate and accumulate contaminants are increasingly the focus of environmental assessments in coastal regions. In order to determine the spatial extent of sources and sinks of sediment contamination on a regional scale, Hornsby Shire Council instigated an investigation of sediment quality in the Lower Hawkesbury River as part of the Estuary Management Program. This study will assist Council in understanding problematic areas and enable prioritisation of remedial actions required to improve sediment quality. These actions will be incorporated within the Lower Hawkesbury Estuary Management Plan and implemented progressively within a 7-10 year timeframe.

Sediment samples were collected at 52 locations within the Lower Hawkesbury River and tributaries using a surface sample grab sampler. These samples were analysed for trace metals (As, Ba, Be, Cd, Cr, Co, Cu, Hg, Mn, Ni, Fe, Pb, Sr, V, Zn), major elements (Al, Ca, Mg), nutrients (total Kjeldahl nitrogen, nitrate and nitrite), total organic carbon, organic contaminants (PCBs, organochlorine pesticides, organophosphorus pesticides, PAHs, TPHs, BTEX) and grain size. During the second stage of the assessment, a further 16 sediment samples were obtained for the determination of tributyltin (TBT) and other organic contaminants, at ultra trace analytical detection limits. Four of the six highest TBT concentrations in sediments were obtained from sampling locations at or near marinas, including the two highest concentrations Organic booster biocides (diuron, chlorothalonil, irgarol and dichlofluanid), which augment copper-based antifoulants since TBT was banned in antifouling paints in 1989, were analysed in sediments from four locations, 3 in close proximity to marinas and one within Refuge and popular mooring and recreational boating site. Analysis for these compounds in sediments is rarely undertaken in Australia and this study represents the first of its kind for sediments in the Lower Hawkesbury River. By sampling at these locations baseline data was obtained to determine the effects of recreational and commercial boating activities on estuarine sediments. Concentrations of diuron up to 40 g/kg were detected in sediments near a marina, but were undetected within Refuge Bay, a popular mooring area, suggesting that there is an impact on sediments in the vicinity of marinas and boat maintenance activity from booster biocides used in antifoulants.

Regionally, the concentrations of contaminants in surficial sediments are generally below ANZECC/ARMCANZ (2000) sediment quality guideline values, indicating that the sediments in the Lower Hawkesbury River are unlikely to pose adverse effects on benthic biota. However, the regional geochemical assessment has confirmed that sediments in the vicinity of marinas and riverside settlements in upper Berowra and Cowan Creeks are contaminated with TBT and other heavy metals and would need to be monitored in future.

Introduction

URS Australia Pty Ltd (URS) was commissioned by Hornsby Shire Council to assess the contaminant status of the estuaries within Hornsby Shire, identify which contaminants are present and establish baseline data on levels of contamination to allow an assessment of the risks posed by contamination to human use and amenity, and ecological values of the Lower Hawkesbury River. The investigation was separated into two stages, namely a Sediment Study (Stage 1) and an Antifoul Study (Stage 2).

A principal aim of the Sediment Study (Stage 1) was to identify locations within the study area that have potentially been impacted by anthropogenic activities, which is reflected by elevated concentrations of organic and inorganic contaminants in surficial sediments. In addition, a comparison of the current levels of sediment contamination with levels established in previous studies was aimed at identifying areas of accumulation and assesses sources of contemporaneous contaminants and potential anthropogenic contributions in the sediments. The subsequent Antifoul Study (Stage 2) investigation was aimed at focussing further sampling at locations that have been identified by the Stage 1 sediment sampling as potential hot spots of sediment contamination and to determine whether or not the use of antifouling products in the Lower Hawkesbury River has resulted in contamination of the estuaries within Hornsby Shire with specific interest in contamination caused by products used as TBT alternatives. In addition, the Antifoul Study (Stage 2) investigation aimed at identifying whether past and present recreational boating activities have resulted in elevated concentrations of tributyltin (TBT) in sediments adjacent to marinas and other areas of urban settlement within the study area. The Antifoul Study (Stage 2) also attempted to identify which contaminants are present in the estuaries within Hornsby Shire as a result of the use of antifouling products and look at a range of commercially available antifouling products and document current research.

Although TBT has been banned as an antifoul compound in paints in NSW since the late 1980s, the legacy of the use of this compound is still likely to represent an environmental risk to benthic biota. In addition, the use of TBT alternative compounds, such as antifoul booster biocides (e.g. diuron, Irgarol 1051, dichlofluanid, chlorothalonil) was assessed at four locations to establish, for the first time, baseline concentrations of these compounds in sediments in the Lower Hawkesbury River.

The study area for the Sediment and Antifoul Study comprised the primary waterways of the Lower Hawkesbury River (upstream limit of Wisemans Ferry to the downstream limit of Croppy Point), Marramarra Creek, Berowra Creek, Cowan Creek, Mooney Mooney and Mullet Creeks. This study area incorporates the two study areas defined in the Berowra Creek and Brooklyn Estuary Process Studies (Figure 1a).

Anthropogenic trace metal contaminants have been assessed in the Hawkesbury River for over 20 years. Early work included the determination of the speciation of iron, copper and zinc in waters of the Hawkesbury River by Pik et al. (1982) and the assessment of heavy metal enrichment in the surface water microlayer of the Hawkesbury-Nepean River system (Barnes et al., 1982). Markich and Brown (1998) conducted a survey of trace metal concentrations in the freshwater reaches of the Hawkesbury River, establishing that concentrations of nutrients, organic carbon and trace metals in the Hawkesbury River increased as a consequence of anthropogenic inputs, particularly point discharges from sewage treatment plants (STPs) as well as diffuse urban and agricultural runoff during storm events. Markich and Brown (1998) established that the temporal variability of trace metals is correlated to the variability in water discharge. An increase in concentrations of trace metals by a factor of up to 2 was correlated with increased water discharge and was cited as evidence of anthropogenic point and diffuses source contributions during high flow events.

Anthropogenic heavy metal contaminants in sediments in the Hawkesbury River have been extensively studied since the mid-1990s and the majority of contaminant investigations of sediments in the Hawkesbury River estuary have been conducted by the Environmental Geology Group (EGG) at the University of Sydney and others (i.e. Parker, 1992; Shotter, 1994; Hardiman and Pearson, 1995; O’Donnell, 1995; Hayes and Buckney, 1998; Birch et al., 1998, 1999; Simonovski et al., 2003). A baseline contaminant survey of the Hawkesbury River estuary in December 1990-January 1991 (EPA, 1996) established the concentrations of a range of organic and inorganic contaminants in sediments of the Hawkesbury River estuary, including the concentrations of TBT.

Concentrations of heavy metals in sediments of the Hawkesbury River are generally close to background, but are substantially elevated in headwaters of Berowra and Cowan Creeks and in southeast Pittwater (Birch et al., 1998, 1999). In addition, high levels of contamination of sediments have been found adjacent to the West Hornsby and Hornsby Heights STPs. Simonowski et al. (2003) reported that sediments in the upper Hawkesbury-Nepean River are not heavily polluted by heavy metals, although elevated concentrations of heavy metals were found in sediments near industrialized areas and STPs.

Methodology

Sediment Study (Stage 1)

Sediment samples were collected at 52 locations, including 19 of the 20 locations that were sampled during the Berowra Creek Estuary Process Study (1998) (Coastal & Marine Geosciences (1998) and the Brooklyn Estuary Process Study (UNSW, 2002). The 33 new sampling locations were selected in collaboration with Hornsby Shire Council (HSC), using existing information on sources of contamination and previous sediment data.

Sediment sampling was undertaken using an EkmanTM grab sampler. Sampling was conducted from a motorized boat supplied by HSC on 13, 14 and 21 December 2006. A Global Positioning System (GPS) was used to locate the sampling locations.

One homogenised sample was collected to a maximum depth of 10 cm at each sampling location. Collection of single samples in the regional assessment of contaminant trends in sediments in the Hawkesbury River estuary has previously shown to be an acceptable and cost-efficient methodology because the small-scale spatial variability of concentrations of contaminants over short distances in the Hawkesbury River is typically lower than the variability over a regional scale, therefore allowing an identification of both regional contaminant trends and contamination point sources in sediment (Birch et al., 1998, 1999, 2001). Following retrieval, samples were photographed and homogenized using a stainless steel spoon and bowl. Homogenized samples were transferred into laboratory-supplied glass jars for geochemical analyses and a plastic bag for grain size analyses. Sample containers were pre-labelled and filled with zero headspace and samples were stored in eskies on ice. Samples collected from each sampling location were given unique sample numbers (H1 to H55, excluding H40 to H42). Samples were delivered to the laboratory within 48 hours of sampling for processing and analysis under chain-of-custody (CoC) requirements. All sample handling and processing were performed to minimize contamination and possible cross-contamination of samples. Nitrile gloves were worn by the sampling personnel.

Sediment samples were analysed by ALS Environmental, a NATA accredited laboratory, for total Kjeldahl nitrogen (TKN), total organic carbon (TOC), nitrate and nitrite (NOX); reactive phosphorus, aluminium, arsenic, barium, beryllium, calcium, cadmium, chromium, cobalt, copper, iron, lead, manganese, mercury, strontium, vanadium, zinc, polychlorinated biphenyls (PCBs), polycyclic aromatic hydrocarbons (PAHs), organochlorine pesticides (OCs), organophosphate pesticides (OPs), total petroleum hydrocarbons (TPHs) (C6-C36) and BTEX. Grainsize was determined by wet sieving (4 fractions: <0.063 mm; 0.063 mm-0.25 mm; 0.25 mm-2 mm; >2 mm).

Six field replicates were collected to assess the reproducibility of the geochemical analyses and the sampling and subsampling methodology and measure the precision of the whole sampling and analysis process (sample collection, sample preparation and sample analysis). Field duplicates were analysed for all analytes and data validation confirmed the suitability of all analytical sample batches for environmental interpretive use.

The regional sediment geochemical data was displayed spatially on a GIS system (MapinfoTM) and the data screened against ANZECC/ARMCANZ (2000) sediment quality guideline values.

Antifoul Study (Stage 2)

Based on the outcomes of the Sediment Study (Stage 1), sixteen priority locations were identified for the Antifoul Study (Stage 2) (Figure 1b). The sampling locations were selected in upper Berowra Creek (H7-H10), Sandbrook Inlet (H17-H18 and H38), Hawkesbury River marina (H37), and Cowan Creek, including Bobbin Head marina (H21), Apple Tree Bay (H22), Waratah Bay (H23), Smiths Creek (H24), Coal and Candle Creek (H25-H26), Cottage Point (H28) and Refuge Bay (H31). These 16 sampling locations are a subset of the sampling locations visited during the Sediment Study (Stage 1) and adequately represent areas of potential sediment contamination. Antifoul booster biocides were analysed in sediments from four locations (H37 – Hawkesbury River marina; H9 - Berowra Creek Marina; H38 - Sandbrook Inlet; H31 - Refuge Bay) by the National Measurement Institute (NMI).

Sediment grab samples up to a depth up to 0.1m from these 16 locations were collected on 26 March 2007, using an EkmanTM grab sampler. Two field replicates were obtained at two sampling locations. Sediment samples were analysed for TBT and organic contaminants (PCBs, PAHs, OCs, OPs, TPHs, BTEX) at ultra trace levels of analytical detection. The date and time of sampling, water depth, latitude and longitude (GPS) and a field description of the sediment type were recorded in the field and a photograph of the sample was taken.

Results

Sediment Study – Stage 1

Sediments were collected in water depths varying from <1 m (Marramarra Creek) to 24 m (Cowan Creek). Gravel (>2 mm) was not present in any sediment sample, although bivalve shell fragments were observed at some locations in the Hawkesbury River. Sediment texture varied from grey to dark grey silty sand to sand (1 sample), clayey sand (6 samples), sandy clay (10 samples) to silty clay (35 samples). Forty-five of the 52 samples contained >50% mud (i.e. <0.063 mm fraction). Sandy sediment was present in the Hawkesbury River main channel and Marramarra Creek. Sediments at 28 sampling locations contained >90% mud in the deeper sections of Berowra Creek, Cowan Creek, Mullet Creek and Mooney Mooney Creek and in sediments of Sandbrook Inlet south of Long Island and in areas of the Upper Hawkesbury River.

The concentration ranges and descriptive statistics of inorganic analytes are summarized in Table 1. The concentrations of some analytes are highly variable, exceeding one order of magnitude (i.e. aluminium, cobalt, copper, lead, manganese, strontium, vanadium, zinc, calcium, TOC and TKN). Distributions of selected heavy metals and TOC in surficial sediments are discussed below.

Copper concentrations were highly variable (<5 mg/kg to 684 mg/kg). However, concentrations of copper >65 mg/kg were only present in sediments at seven locations in headwaters of Cowan Creek, Berowra Creek and at Cottage Point. The concentrations of copper decreased markedly from 82 mg/kg to 15 mg/kg downstream of Berowra Creek. Similarly, the concentration gradient in Cowan Creek was from 68 mg/kg to 19 mg/kg. Concentrations of copper in the main Hawkesbury River channel were generally <20 mg/kg and a spatial concentration gradient was not apparent.

Concentrations of lead were highly variable (8 mg/kg to 179 mg/kg) and lead concentrations >50 mg/kg were present in sediments at ten sampling locations in Berowra Creek, Cowan Creek (including Coal and Candle Creek and Akuna Bay) and Hawkesbury River marina. Similar to the spatial trends in copper, lead concentrations decreased substantially downstream from the headwaters of Berowra and Cowan Creeks towards the main Hawkesbury River channel. Concentrations of lead in sediments of the Hawkesbury River were generally <30 mg/kg and did not display a spatial trend.

Concentrations of mercury varied from <0.1 mg/kg to 0.9 mg/kg and sediments from eight sampling locations exceeded the analytical detection limit (0.1 mg/kg) (Cowan Creek; Hawkesbury River marina; H15 and H36 in the Hawkesbury River).

Concentrations of zinc in sediments varied from 26 mg/kg to 412 mg/kg and the highest concentration of zinc was present in sediment at Bobbin Head marina (H21). However, all other samples contained <200 mg/kg zinc. There is a pronounced decrease in the concentrations of zinc from the headwaters of Berowra Creek and Cowan Creek downstream towards the main Hawkesbury River channel. In Berowra Creek the zinc concentrations decreased from 183 mg/kg to 72 mg/kg. Similarly, the concentrations of zinc in sediments in Cowan Creek decreased from 412 mg/kg to 72 mg/kg. In contrast, the concentrations of zinc in sediments of the main Hawkesbury River channel were generally <90 mg/kg.

TOC contents in sediment varied from 0.9% to 13%. Organic-rich sediments (>10% TOC) were present in Berowra Creek and southwest of Milson Island in the Hawkesbury River. A distinct regional trend in the TOC contents of sediments was present in the Hawkesbury River, where sediments contained 5-10% TOC, decreasing east of Mooney Mooney to <5% TOC. The majority of organic contaminants were below analytical detection limits in all or most samples.

Antifoul Study – Stage 2

The concentrations of TBT were determined in sediments from sixteen locations in upper Berowra Creek, Sandbrook Inlet, Hawkesbury River marina and Cowan Creek, including Bobbin Head marina, Apple Tree Bay, Waratah Bay, Smiths Creek, Coal and Candle Creek, Cottage Point and Refuge Bay.

Concentrations of TBT exceeded the analytical detection limit (0.5 gSn/kg) in all samples from all 16 locations, and varied from 1.4 gSn/kg to 125 gSn/kg. The concentrations of TBT in sediments in Berowra Creek decreased from 6.6 gSn/kg at sampling locations H10 to 1.6 gSn/kg at H7. Sediments in Sandbrook Inlet contained TBT concentrations that varied from 1.6 gSn/kg to 4.5 gSn/kg. The concentration of TBT in sediments at Hawkesbury River marina (H37) was 10.4 gSn/kg. In Cowan Creek and its tributary creeks, the concentrations of TBT were also highly variable, with concentrations of <2 gSn/kg at Waratah Bay, Smiths Creek and Refuge Bay, compared to concentrations of 22.9 gSn/kg and 114 gSn/kg at Akuna Bay marina, 41.5 gSn/kg at Cottage Point, 10.4 gSn/kg at Apple Tree Bay and 125 gSn/kg at Bobbin Head marina (Figure 2). Four of the six highest TBT concentrations in sediments were obtained from sampling locations at or near marinas, including the two highest concentrations (Figure 2). In contrast, the five lowest concentrations of TBT of <2 gSn/kg were all from locations that were not adjacent to marinas.

The concentrations of BTEX, OPs, OCs PCBs and TPHs were below the analytical detection limit in all samples, except for 4.4’-DDE in samples from Berowra Creek (up to 1.1 g/kg) and upper Cowan Creek (1.0 g/kg) and PCBs at two locations.

Concentrations of PAH compounds exceeded the analytical detection limit (10 g/kg) in one or more samples for most compounds (Table 2). Low molecular weight PAHs varied from 65 g/kg to 420 g/kg and high molecular weight PAHs varied from 165 g/kg to 3,960 g/kg. Total PAHs varied from 405 g/kg to 7,040 g/kg. Total PAH concentrations below 1,000 g/kg were found in sediments from Cowan Creek (Smiths Creek, Refuge Bay, Akuna Bay marina and Cottage Point). In contrast, total PAH concentrations exceeding 2,000 g/kg were present in sediments from Sandbrook Inlet, Hawkesbury River marina, Cowan Creek marina, Apple Tree Bay, and Akuna Bay marina.

Concentrations of organic booster biocide compounds diuron, chlorothalonil, Irgarol and dichlofluanid in sediments at Berowra Creek marina, Refuge Bay, Hawkesbury River marina and Sandbrook Inlet were below the analytical detection limit (0.1 mg/kg) in all four samples. However, inspection of chromatograms allowed a greater sensitivity of analysis with a detection limit of 0.001 mg/kg and revealed measurable concentrations of diuron in sediments at three of the four sampling locations (9 g/kg (Hawkesbury River marina), 10 g/kg (Sandbrook Inlet), 30 g/kg (Berowra Creek marina), and 40 g/kg (replicate sample from Berowra Creek marina). There was no visible chromatographic peak for diuron in the sediment from Refuge Bay (<1 g/kg).

Discussion

Sediment Quality in the Lower Hawkesbury River

The regional sediment geochemical data was screened against ANZECC/ARMCANZ (2000) sediment quality guideline values (ISQG-L (Trigger Value) and ISQG-H (High)) to assess whether sediments may require additional investigations. If the Trigger Value is not exceeded, it is unlikely that there are ecological effects on organisms inhabiting that sediment. If the Trigger Value is exceeded, management (including remedial) action may be required, or additional site-specific studies may be conducted to determine whether this exceedance poses a risk to the ecosystem.

Four inorganic contaminants (arsenic, copper, lead and mercury) exceeded the ISQG-L sediment quality guideline value in at least one sample, whereas the ISQG-H value was exceeded for copper and zinc at Bobbin Head marina. The number of exceedances of ISQG-L values at individual locations varied from one (13 sampling locations), two (4 sampling locations) to three (2 sampling locations), with zero exceedances at 33 sampling locations.

Three ISQG-L value exceedances were present at Bobbin Head and Akuna Bay marinas, suggesting that anthropogenic contributions from marina activities may have resulted in a contamination of surficial sediments. Two ISQG-L value exceedances were present at Berowra Creek marina, Apple Tree Bay, Cottage Point kiosk and Hawkesbury River marina, whereas single ISQG-L exceedances were present near Berowra Creek marina, in the Hawkesbury River south of Spectacle Island, Sandbrook Inlet, Mullet Creek, Cowan Creek and the Hawkesbury River south of Dangar Island.

The regional distribution of the number ISQG-L value exceedances is shown in Figure 3. There were no exceedances in sediments in the Hawkesbury River between Wisemens Ferry and Mooney Mooney. Sediments in Marramarra Creek and Berowra Creek between Calabash Bay and at the confluence with the Hawkesbury River also did not exceed any ISQG-L values. However, exceedances of sediment quality guideline values were present in sediments in the headwaters of Berowra Creek, Sandbrook Inlet, and in Cowan Creek, including Bobbin Head, Apple Tree Bay, Waratah Bay, Smiths Creek, Akuna Bay, Cottage Point and Refuge Bay.

The sediment geochemical data for TBT and other organic compounds assessed during the Antifoul Study (Stage 2) were also screened against ANZECC/ARMCANZ (2000) sediment quality guideline values (normalized to 1% TOC). Normalized concentrations of PAHs and TBT were below the ISQG-L values at all 16 sampling locations, except for TBT (5 gSn/kg) at four locations at Cowan Creek marina, Akuna Bay marina (2 locations) and Cottage Point kiosk. Exceedances of ISQG-L values were therefore limited to locations near marinas in Cowan Creek and Coal and Candle Creek and the kiosk at Cottage Point, where a high density of recreational boating activity exists.

Concentrations of PAHs (normalized to 1% organic carbon) did not exceed the sediment quality guideline values at any sampling location. However, PAHs are ubiquitous in sediments in the Lower Hawkesbury River and most PAH compounds that were assessed were present at concentrations that exceed five times the analytical detection limit. The presence of PAHs in these sediments is likely to be of anthropogenic origin, in particular at locations adjacent to increased recreational boating activity. However, PAHs are also produced by combustion of wood during bushfires and the widespread distribution of these compounds in sediments in the Lower Hawkesbury River and tributary creeks suggests that natural contributions from burnt organic matter originating from bushfires over long periods of time may have contributed to the regional presence of these compounds in surficial sediments.

Potential Sources of Contaminants to Sediments

Concentrations of inorganic and organic contaminants in sediments in the Lower Hawkesbury River, including Cowan Creek, Berowra Creek, Mooney Mooney Creek, Mullet Creek and Marramarra Creek are elevated at several sampling locations near potential point sources (e.g. marinas). The regional contaminant distributions identify point sources, in particular near centres of recreational boating activities. These results are consistent with previous sediment investigations in the Lower Hawkesbury River (Birch et al., 1998, 1999) which have highlighted high density boating activities as significant but localized contributors of contaminants to surficial sediments. However, stormwater and urban runoff from diffuse sources within the Berowra Creek and Cowan Creek catchments may be overprinted by local sources, such as marinas.

Upper Cowan Creek and Berowra Creek sediments display elevated concentrations of inorganic and organic contaminants which demonstrate the potential for water-related recreational facilities to affect pristine environments. Increased urbanisation and industrialisation in the upper Berowra Creek catchment provide a source of heavy metals and nutrients, possibly from STP contributions and stormwater drainage. Sewage effluent discharges from isolated urban areas (Berowra Waters, Brooklyn) on the Hawkesbury River are also likely to contribute to the observed elevated concentrations of heavy metals (i.e. copper, lead, mercury, zinc).

Temporal Changes in Contaminant Concentrations

The temporal changes in the concentrations of contaminants were assessed for selected contaminants at 18 sampling locations which were revisited in the Sediment Study (Stage 1). There are some factors which may confound a direct comparison of the previous chemical data with the data obtained in the current investigation. During the 2002 Brooklyn Estuary Process Study triplicate sediment samples were collected compared to single sediment samples that were collected in the current investigation. Analytical data for the three sediment samples was therefore compared to the data from the single sample obtained in the current investigation (2007 data).

Geochemical data that have been compared between the previous Brooklyn Estuary Process Study (2002) and the Berowra Creek Estuary Process Study (1998) and the current investigation, include silt+clay content, arsenic, chromium, copper, lead, nickel, zinc, TOC and TKN (Table 3).

The silt+clay content was generally similar at many of the locations sampled in 1998/2002 and 2007 (Figure 4). The concentrations of metals in whole sediment and normalized to mud content (i.e. <0.063 mm fraction) in the upper sections of Berowra Creek from Bobbin Head to Calabash Bay are substantially greater in 2007 compared to 1998/2002 for a number of analytes, including copper, chromium, lead and zinc (e.g. Cu in Figures 5 and 6). In contrast, the relative differences in the concentrations of nickel, arsenic and TKN in upper Berowra River are minimal, suggesting no measurable change in the surficial sediment concentrations of these analytes between 1998 and 2007 and confirming a contribution of anthropogenic trace metals to sediments in upper Berowra Creek.

In contrast, TOC contents have substantially decreased in upper Berowra Creek between 1998 and 2007 from 7-15% to 3-6%, respectively. Sediments downstream of Calabash Bay contained between 6-13% TOC in 2007 compared to <3% in 1997, which may be due to variations in organic carbon contributions to sediments throughout Berowra Creek.

Potential Sources and Biological Effects of TBT

Tributyltin (TBT) is a biocide and catalyst used worldwide. TBT compounds have particularly been used as biocides in antifouling paints and wood preservatives (Hagger et al., 2005). The need for effective antifoulants, which prevent the settlement and growth of marine organisms on submerged structures, such as buoys, fish cages and ship’s hulls, is recognised universally (Evans et al., 2000; Konstantinou and Albanis, 2004). For many years, TBT compounds were the most widely used active ingredients in antifoul paint formulations. However, use of TBT has been regulated internationally since 1990 (1989 in New South Wales, Australia – Gibson and Wilson, 2003) due to its severe impact on the aquatic ecosystem (Fent, 1996) and the demonstrated effects of TBT on the disruption of the endocrine system by mimicking or inhibiting the action of gonadal steroid hormones, oestradiol and testosterone (Makita and Omura, 2006). Leachate of TBT compounds has contaminated both marine and freshwater habitats and it has been considered one of the most toxic agents entering the environment (Hagger et al., 2005).

The harmful environmental effects of organotin compounds were recognized by the International Maritime Organization (IMO) in 1989. In 1990 IMO’s Marine Environment Protection Committee (MEPC) adopted a resolution which recommended that Governments adopt measures to eliminate the use of anti-fouling paint containing TBT on non-aluminium hulled vessels of <25 m in length and eliminate the use of anti-fouling paints with a leaching rate of >4 g/d of TBT.

Prior to the ban on the use of TBT as an antifouling compound, the source of alkyltins (mono-, di- and tributyltin) to the waterways of the Lower Hawkesbury River were chiefly from the leaching of antifouling paints from locally moored commercial and recreational vessels and, to a lesser extent, leaching from PVC pipes used in domestic and industrial plumbing (EPA, 1996). However, elevated concentrations of alkyltins in estuarine sediments are principally associated with commercial marinas and boatyard facilities in areas of limited water movement or tidal flushing (SPCC, 1988). TBT compounds tend to have medium range water solubility and tend to sorb to particulate matter and suspended solids. These compounds are therefore settling with the particulate matter and, as a result, increase the concentrations of TBT in the sediments (Dowson, 1992). The water solubility of TBT is strongly affected by pH. TBT’s solubility ranges from 0.75 to 31 g/kg over the pH-range of 2.6 to 8.1 (Uhler et al., 2000).

The use of TBT for antifouling was regulated from 1989 in NSW (Gibson and Wilson, 2003). TBT is banned from use on small vessels <25m in length and is being phased out on larger and international vessels. However, despite the partial ban on the use of TBT in Australia, a survey of imposex in Thais orbita (Neogastropoda) along the NSW coast found imposex was still widespread 10 years after the introduction of the ban, in particular within harbour/bay areas, where contamination “hotspots” are still present and where physical remobilization and dispersion processes may be less pronounced compared to high-energy coastal areas (Gibson and Wilson, 2003).

Importantly, the presence of TBT in sediments is not necessarily linked to silts and muds because TBT may occur in flakes that may be deposited in higher energy sandy areas, such as shipping channels. Therefore, TBT may potentially be present in measurable quantities in sediments in areas that may not otherwise exhibit elevated concentrations of inorganic analytes or organic compounds.

The USEPA (2003) has undertaken a review on TBT which contains a very extensive review of biological effects of TBT. It has been proposed by CSIRO to the Department of Environment and Water Resources that these guidelines be adopted for TBT in Australia (S. Simpson, CSIRO, pers. comm., 2007).

Temporal Variations of TBT in Hawkesbury River Sediments

A baseline contaminant survey of the Hawkesbury River estuary in December 1990-January 1991 (EPA, 1996) established the concentrations of alkyltin compounds in sediments at a number of locations within the Hawkesbury River estuary. The EPA (1996) baseline survey of sediments involved the collection of 158 surface sediment grab samples from 102 sampling locations and analysis for trace metals, organochlorine compounds, nutrients, TBT, grain size and organic matter. The concentrations of TBT in the EPA (1996) investigation varied from 0.1 gSn/kg to 9.6 gSn/kg, with a mean concentration of 0.9 gSn/kg. However, the highest concentrations of TBT in sediments were found in Pittwater, an area which was outside the area studied in the 2007 Antifoul Study (Stage 2). The highest mean concentrations of TBT in sediments reported in EPA (1996) were in the eastern end of Sandbrook Inlet (2.1 gSn/kg), upper Coal and Candle Creek (near Akuna Bay marina) (1.1 gSn/kg) and upper Cowan Creek (2.8 gSn/kg).

A significant difference between the EPA (1996) investigation and the 2007 Antifoul Study (Stage 2) is that the sampling locations were selected based on a hierarchical approach that was based on the outcomes of the Sediment Study (Stage 1) and the inferred proximity to anthropogenic sources of contaminants, in particular TBT, from marinas. The concentrations of TBT in sediments at the 16 Antifoul Study (Stage 2) sampling locations within the Lower Hawkesbury River, including Cowan Creek and Berowra Creek, are substantially higher at several sampling locations adjacent to marinas compared to locations elsewhere. This corresponds to elevated concentrations of heavy metals at locations in the vicinity of marinas, as shown in the Sediment Study (Stage 1) and exemplified by the positive correlation between TBT and copper in sediments (Figure 7). A pronounced regional distribution of concentrations of TBT in sediments identifies local point sources within the Lower Hawkesbury River, in particular in areas of high boating activity. These results are consistent with previous investigations for other organic and inorganic contaminants in Lower Hawkesbury River sediments (EPA, 1996, Birch et al., 1998, 1999), which have also highlighted high density boating activities as significant but localized contributors of contaminants to sediments.

Sediments in upper Cowan Creek and upper Berowra Creek display elevated concentrations of inorganic and organic contaminants, thereby demonstrating the potential for water-related commercial and recreational facilities to affect pristine environments. Although the Antifoul Study (Stage 2) did not address the potential for bioavailability of TBT, it has been shown that TBT is chemically mobile and therefore bioavailable to aquatic biota, an aspect that may require further investigations.

The spatial distribution of TBT and the exceedances of ANZECC/ARMCANZ (2000) sediment quality guideline values have established a distinct relationship between proximity to boating centres (marinas) and elevated concentrations of TBT in surficial sediments (Figure 8). The outcomes of the Antifoul Study (Stage 2) may be summarized as follows:

- Brooklyn/Sandbrook Inlet: 4 sampling locations; TBT concentrations varied from 1.6-10.4 gSn/kg; and

- Cowan Creek: 8 sampling locations; TBT concentrations varied from 1.4-125 gSn/kg.

Temporal variations in the concentrations of TBT in sediments in Berowra Creek, Sandbrook Inlet/Brooklyn and Cowan Creek between 1990/91 and 2007 are likely due to the different sampling methodologies used in the two studies. The EPA (1996) investigation utilized a random grid sampling methodology, whereas the 2007 Antifoul Study (Stage 2) followed a hierarchical sampling approach with the intention to establish contaminant concentrations at likely point sources of contaminants (i.e. marinas and urban developments). The outcomes of these substantially different approaches are therefore not directly comparable. However, TBT has been banned in NSW since 1989 and it would be expected that substantially lower concentrations of TBT are present in surficial sediments in 2007 compared to 1990-1991. Concentrations of TBT in sediments collected in 2007 have shown to decrease with increasing distance from potential point sources (e.g. downstream of Berowra Creek marina) and are generally higher near marinas compared to sediments in remote areas (e.g. Refuge Bay, Smiths Creek and Waratah Bay).

Elevated concentrations of TBT in surficial sediments near marinas suggest that the deposition of present-day TBT-free particulate matter is consistently bioturbated and mixed with underlying TBT-enriched sediments. This continuous mixing process results in only slight dilutions near existing recreational boating facilities, which have been operating for many years prior to the implementation of the TBT ban, and where the presence of high concentrations of TBT in the sediments is likely to be a legacy of past shore-based activities and hull cleaning operations at slipways.

An important finding is the presence of TBT at least three times greater than the analytical detection limit (0.5 gSn/kg) at all sixteen sampling locations, despite the absence of any apparent point sources of TBT at some locations (e.g. Berowra Creek (Calabash Bay); Waratah Bay; Smiths Creek; Refuge Bay). This suggests a regional presence of TBT in sediments that may be the result of resuspension and downstream transport of particulates from the upper reaches of the two major Creeks and the dispersion and transport of TBT from recreational vessel movements into previously pristine areas. Although concentrations of <5 gSn/kg are unlikely to result in adverse biological effects in benthic biota, the widespread presence of TBT, albeit at low concentrations, is an indication of an enlarged anthropogenic footprint beyond the immediate proximity of marinas, due to increasingly common boating activity in this region.

Previous investigations to characterise the physical and geochemical characteristics of sediments to be removed from Hawkesbury River marina during maintenance dredging included the determination of TBT and have been discussed in Patterson Britton (2003). Dames and Moore (1999) also reported that isolated areas of TBT exceedances in sediments above the then applicable ANZECC (1992) interim sediment quality guideline values of 5 gSn/kg and 37 gSn/kg existed in the area of assessment at Brooklyn Harbour.

A sediment investigation by Patterson Britton (2001) detected TBT at concentrations above the ANZECC/ARMCANZ (2000) ISQG-L sediment quality guideline value (8-56 gSn/kg) at five of ten sampling locations. The high concentrations of TBT were confirmed by the result of the 2007 investigation, which established a TBT concentration of 10.4 gSn/kg at the Hawkesbury River marina.

Organic Paint Booster Biocides

Although copper is effective against the majority of fouling organism, several algal biofoulers show marked physiological tolerance to copper (Voulvoulis et al., 2000; Amog Consulting, 2002). A booster biocide is therefore usually incorporated in copper based antifouling paints to control copper-resistant species. In the United Kingdom, for example, twelve booster biocides are used as active ingredients in approved antifouling products: Irgarol 1051, diuron, Kathon 5287 (Sea-Nine 211), maneb, zineb, thiram, ziram, chlorothalonil, dichlofluanid, TMTB, TCMS pyridine, and zinc pyrithione (Amog Consulting, 2002). Irgarol 1051 is active against macroalgae, whereas chlorothalonil shows more broad-spectrum activity. Biodegradability can also vary considerably, ranging from readily biodegradable (Sea-Nine 211) to non-biodegradable (diuron and Irgarol 1051). Anaerobic half-lives of <0.5 days were calculated for chlorothalonil and dichlofluanid, between 14 and 35 days for diuron and over 226 days for Irgarol 1051 (Thomas et al., 2003), confirming the greater non-biodegradability of diuron and Irgarol 1051 relative to chlorothalonil and dichlofluanid. Therefore, the release of booster biocides associated with paint particles into marinas has the potential to lead to their accumulation unless activities such as hull cleaning are strictly regulated.

Although TBT still exceeds risk quotients by the greatest margins in environmental risk assessment of biological effects in the United Kingdom, widespread effects due to Irgarol 1051, and less so diuron, cannot be ruled out, particularly if use patterns change (Thomas et al., 2001). Khandeparker et al. (2005) reported that the larval development and post-settlement metamorphosis of the barnacle Balanus albicostatus Pilsbry (Crustacea: Cirripedia) and the larval metamorphosis of a serpulid polychaete, Pomatoleios kraussii Baird was affected when exposed to Irgarol 1051, due to the promotory biofilms becoming toxic. Similarly, exposure of tropical seagrasses to diuron resulted in photosynthetic stress and a decline in the effective quantum yield within hours of exposure in Cymodocea serrulata, Halophila ovalis and Zostera capricorni (Haynes et al., 2000). It is therefore likely that adverse biological effects from the use of organic booster biocides in antifoul paints may also be present in areas of high recreational boating activity in the Lower Hawkesbury River.

The availability of booster biocide compounds in registered antifoulant products available in Australia is summarized in Table 4. This summary neither complete nor exhaustive, and provides only a general indication of the prevalence of organic booster biocides in antifoul products in Australia.

Organic booster biocides are not routinely assessed in environmental media in Australia, although some studies have been conducted in Australia (Konstantinou and Albanis, 2004 and references therein). Given the relative scarcity of data and analytical costs and uncertainties, an assessment of organic booster biocides necessarily required a compromise approach both in terms of the number of sampling locations and the number of analytes to be assessed. Diuron and Irgarol 1051 were included in the suite of organic booster biocide analytes due to their relative non-biodegradability (Amog Consulting, 2002). Although Irgarol 1051 is not registered with the National Registration Authority (NRA) in Australia (Amog Consulting, 2002), precluding its use in marine and other products, the presence of this compound in sediments would indicate that contributions from overseas vessels may be responsible. In addition, dichlofluanid and chlorothalonil were considered to be of value in the assessment of sediments at some locations in the Lower Hawkesbury River because these compounds are present in registered antifoul products in Australia (Table 4).

The novel assessment of sediments for organic booster biocide compounds in the Lower Hawkesbury River estuary has provided an opportunity to establish a baseline for comparisons with potential future studies involving these compounds, in particular when more detailed data on the effects of these compounds on benthic organisms become available through overseas and local studies.

Organic Paint Booster Biocides in Lower Hawkesbury River Sediments

Although the analytical detection limits of dichlofluanid, Irgarol 1051, diuron, and chlorothalonil in the international scientific literature in both water and sediments are routinely reported in the low g/kg range (1-10 g/kg), the quantitative analytical determination of these compounds is not routinely performed by the major commercial laboratories in Australia. Analysis of the detailed mass spectrometry data from the sample analyses provided a more sensitive analytical detection limit that resulted in the detection of diuron concentrations up to 40 g/kg. There was no detectable concentration of diuron in sediment sampled at Refuge Bay (<1 g/kg).

Based on chemical data reported in the scientific literature, booster biocide concentrations >100 g/kg in sediments have previously been reported for Irgarol 1051 (<1-1,011 g/kg) (Boxall et al., 2000; Konstantinou and Albanis, 2004), diuron (<0.4-1,420 g/kg) (Thomas et al., 2000), dichlofluanid (<1.6-688.2 g/kg) (Voulvoulis et al., 2000) and chlorothalonil (16-165 g/kg) (Albanis et al., 2002). The absence of Irgarol 1051, dichlofluanid, and chlorothalonil in Hawkesbury River sediments at concentrations <100 g/kg is not an indication of the absence of these compounds but it suggests that if these compounds are present, the concentrations are likely to be below concentrations found in sediments at European ports and marinas.

Diuron is the most common of the four booster biocide compounds assessed in the current investigation in Australia (Table 4), which corresponds to the presence of this compound in sediments at concentrations up to 40 g/kg. In contrast, dichlofluanid and chlorothalonil are registered in only one antifoul product in Australia and Irgarol 1051 is not legally registered in any antifoul compounds, which is confirmed by the absence of these compounds in sediments at the four locations sampled in the Lower Hawkesbury River.

Diuron concentrations are greater than the concentrations of TBT in corresponding samples at Berowra Creek marina but similar at Hawkesbury River marina and Sandbrook Inlet. In contrast, diuron was not detected at Refuge Bay, where the concentration of TBT was also low (3 gSn/kg) (Table 5).

Low concentrations of diuron and TBT in sediments in Refuge Bay confirm that sediments at that location are uncontaminated, which is expected due to the absence of marinas and only temporary moorings for small recreational vessels in the Bay. In contrast, TBT concentrations up to 10.4 gSn/kg and diuron concentrations up to 40 g/kg at the other three locations correspond to substantially higher boating activity at marinas in upper Berowra Creek, Sandbrook Inlet and Long Island jetty (Brooklyn). However, diuron concentrations up to 40 g/kg in sediments in the Lower Hawkesbury River are generally lower compared to concentrations in sediments at marinas and in estuaries in the United Kingdom (e.g. Orwell marina: <12-395 g/kg (Boxall et al., 2000); Southampton Water: <100-1,420 g/kg (Thomas et al., 2000)) and Spain (e.g. Ebre Delta ports: <0.4-100 g/kg (Martinez and Barceló, 2001)).

Conclusions

Concentrations of arsenic, copper, lead, mercury and zinc exceed ANZECC/ARMCANZ (2000) sediment quality guideline values in sediments of upper Berowra Creek, Cowan Creek, Sandbrook Inlet, mainly adjacent to marinas, and at some locations in the Hawkesbury River (southwest of Spectacle Island and south of Dangar Island). Nineteen of 52 sampling locations exceed at least one sediment quality guideline value. The highest number (3) of sediment quality guideline value exceedances was present at Bobbin Head marina and Akuna Bay marina, likely due to anthropogenic contributions from marina activities. Concentrations of organic compounds are below the analytical detection limits in sediments at most sampling locations.

Whole sediment concentrations of copper, chromium, lead and zinc are higher in upper Berowra Creek sediments in 2007 compared to 1998, possibly reflecting increased contaminant contributions to receiving waters from the upper Berowra Creek catchment. In contrast, TOC contents in sediments are lower in upper Berowra Creek sediments when compared over the same 9 year period, which suggests that elevated contaminant concentrations in upper Berowra Creek sediments may be related to sources other than stormwater discharge.

The Antifoul Study (Stage 2) confirmed that concentrations of TBT (normalized to 1% TOC, as required by ANZECC/ARMCANZ (2000)) in sediments exceed ANZECC/ARMCANZ (2000) sediment quality guideline values at four locations near Akuna Bay marina, Bobbin Head marina and other points of high recreational boating activity (i.e. Cowan Creek – Cottage Point). TBT is widespread in the Hawkesbury River estuary, albeit at low concentrations in most areas. Concentrations of TBT exceeded 1.4 gSn/kg in sediments at each sampling location, including locations remote from marinas (i.e. Calabash Bay, Refuge Bay and Smiths Creek).

TBT concentrations in sediments at marinas and in areas of high recreational boating activity in the Lower Hawkesbury River are substantially lower compared to many other harbours and coastal areas worldwide, indicating that antifoul-related contamination is comparatively minor and localized to areas of known boating activity. Concentrations of organic compounds (i.e. TPH, OP pesticides, BTEX, OC pesticides, and PCBs) in surficial sediments are generally very low. However, PAH compounds are generally ubiquitous in sediments in the Lower Hawkesbury River and possibly due, in part, to natural PAH contributions from burnt organic matter generated during bushfires.

Organic booster biocides Irgarol 1051, chlorothalonil and dichlofluanid have not been detected in sediments in the Lower Hawkesbury River at an analytical detection limit of 100 g/kg. However, diuron is present at concentrations up to 40 g/kg at Berowra Creek marina, Hawkesbury River marina and Sandbrook Inlet), suggesting that these compunds (and other antifoul compounds not assessed in the current investigation (e.g. zinc pyrithione, thiram) may be present at lower concentrations.

Acknowledgements

URS Australia Pty Ltd acknowledges Hornsby Shire Council’s support and project funding (Project No. Q27/2006) and appreciates the reviews of progress reports by Dr Ross McPherson and members of the Brooklyn Estuary Management Committee at meetings in February and July 2007.

References

Albanis, T., Lambropoulou, D., Sakkas, V., and Konstantinou, I. (2002). Antifouling paint booster biocide contamination in Greek marine sediments. Chemosphere, 48, 475-485.

Amog Consulting (2002). Hull Fouling as a vector for the translocation of marine organisms. Phase 3 – the significance of the prospective ban on tributyltin antifouling paints on the introduction & translocation of marine pests in Australia. Report No. 2 to Department of Agriculture, Fisheries and Forestry, Australia, April 2002.

ANZECC/ARMCANZ (2000). Australian and New Zealand Guidelines for Fresh and Marine Water Quality, Australian and New Zealand Environment and Conservation Council/Agriculture and Resource Management Council of Australia and New Zealand, October 2000.

Barnes, R.K., Batley, G.E. and Sharp, J.H (1982). Heavy metal enrichment in the surface microlayer of the Nepean-Hawkesbury river system. Australian Journal of Marine and Freshwater Research, 33, 917-930.

Birch, G.F., Shotter, N. and Steetsel, P. (1998). The environmental status of Hawkesbury River sediments. Australian Geographical Studies, 36, 37-57.

Birch, G.F., Eyre, B.D. and Taylor, S.E. (1999). The use of environmental impact on a large coastal catchment – the Hawkesbury River system. AGSO Journal of Australian Geology & Geophysics, 17, 175-191.

Birch, G.F, Taylor, S.E. and Matthai, C. (2001). Small-scale spatial and temporal variance in the concentration of heavy metals in aquatic sediments: a review and some new concepts. Environmental Pollution, 113, 357-372.

Boxall, A., Comber, S., Conrad, A., Howcraft, J. and Zaman, N. (2000). Inputs, monitoring and fate modelling of antifouling biocides in UK estuaries. Marine Pollution Bulletin, 40, 898-905.

Coastal & Marine Geosciences (1998). Berowra Creek Estuary Process Study Technical Report : Sediment characteristics and processes. Coastal & Marine Geosciences Geological and Environmental Consultant, Sydney.

Dames and Moore (1999). Brooklyn Harbour Dredging and Reclamation – Options Discussion Paper. Report prepared for Hornsby Shire Council.

Dowson, P.H., Bubb, J.M. and Lester, J.N. (1992). Organotin distribution in sediments and waters of selected east coast estuaries in the UK. Marine Pollution Bulletin, 24, 492-498.

EPA (1996). A baseline contaminant survey of the Hawkesbury River Estuary: December 1990-January 1991. New South Wales Environment Protection Authority, EPA Report No. 94/24.

Evans, S.M., Birchenough, A.C. and Brancato, M.S. (2000). The TBT ban: out of the frying pan into the fire? Marine Pollution Bulletin, 40, 204-211.

Fent, K. (1996). Ecotoxicology of organotin compounds. Critical Reviews in Toxicology, 26, 1-117.

Gibson, C.P. and Wilson, S.P. (2003). Imposex still evident in eastern Australia 10 years after tributyltin restrictions. Marine Environmental Research, 55, 101-112.

Hagger, J.A., Depledge, M.H. and Galloway, T.S. (2005). Toxicity of tributyltin in the marine mollusc Mytilus edulis. Marine Pollution Bulletin, 51, 811-816.

Hardiman, S. and Pearson, B. (1995). Heavy metals, TBT and DDT in the Sydney Rock Oyster (Saccostrea commercialis) sampled from the Hawkesbury River estuary, NSW, Australia. Marine Pollution Bulletin, 30, 563-567.

Hayes, W.J. and Buckney, R.T. (1998). Chemical characteristics of sediments and interstitial waters in some small Hawkesbury Sandstone freshwater streams of the Sydney area. Lakes & Reservoirs: Research and Management, 3, 165-178.

Haynes, D., Ralph, P., Prange, J. and Dennison, B. (2000). The impact of the herbicide diuron on photosynthesis in three species of tropical seagrass. Marine Pollution Bulletin, 41, 288-293.

Khandeparker, L., Desai, D. and Shirayama, Y. (2005). Larval development and post-settlement metamorphosis of the barnacle Blanus albicostatus Pilsbry and the serpulid polychaete Pomatoleios kraussii Baird: Impact of a commonly used antifouling biocide, Irgarol 1051. Biofouling, 21, 169-180.

Konstantinou, I.J. and Albanis, T.A. (2004). Worldwide occurrence of antfouling paint booster biocides in the aquatic environment: a review. Environment International, 30, 235-248.

Makita, Y. and Omura, M. (2006). Effects of perinatal combined exposure to 1,1-dichloro-2,2 bis (p-chlorophenyl) ethylene and tributyltin on male reproductive system. Basic and Clinical Pharmacology & Toxicology, 99, 128-132.

Markich, S.J. and Brown, P.L. (1998). Relative importance of natural and anthropogenic influences on the fresh surface water chemistry of the Hawkesbury-Nepean River, south-eastern Australia. Science of the Total Environment, 217, 201-230.

O’Donnell, M. (1995). Heavy metal contamination in the Hawkesbury River between Windsor and Wisemans Ferry. Unpubl. B.Sc. (Hons) thesis, Department of Geology and Geophysics, The University of Sydney, Sydney.

Parker, C. (1992). Heavy metals in the sediments of the Hawkesbury Valley: an assessment of the spatial and temporal distribution. Unpubl. B.Sc. (Hons) thesis, Department of Geography, The University of Sydney, Sydney.

Patterson Britton (2001). Brooklyn Boat Harbour Dredging and Reclamation Vibrocoring and Sediment Testing Exercise. Report prepared by Patterson Britton and Partners for Hornsby Shire Council.

Patterson Britton (2003). Brooklyn Harbour Maintenance Dredging. Environmental Impact Statement prepared for Hornsby Shire Council by Patterson Britton and Partners, Issue No. 2, April 2003.

Pik, A.J., Eckert, J.M. and Williams, K.L. (1982). Speciation of iron, copper and zinc in the Hawkesbury River. Australian Journal of Marine and Freshwater Research, 33, 971-977.

Shotter, N. (1994). Anthropogenic impacts on the central Hawkesbury River, part of a large NSW estuary. Unpubl. B.Sc. (Hons) thesis, Department of Geology and Geophysics, The University of Sydney, Sydney.

Simonovski, J., Owens, C. and Birch, G. (2003). Heavy metals in sediments of the Upper Hawkesbury-Nepean River. Australian Geographical Studies, 41, 196-207.

Thomas, K., Blake S. and Waldock, M. (2000). Antifouling paint booster biocide contamination in UK marine sediments. Marine Pollution Bulletin, 40, 739-745.

Thomas, K.V., Fileman, T.W., Readman, J.W. and Waldock, M.J. (2001). Antifouling paint booster biocides in the UK coastal environment and potential risks of biological effects. Marine Pollution Bulletin, 42, 677-688.

Thomas, K., McHugh, M., Hilton, M. and Waldock, M. (2003). Increased persistence of antifouling paint biocides when associated with paint particles. Environmental Pollution, 123, 153-161.

Uhler, A., Stout, S., McCarthy, K. and Emsbo-Mattingly, S. (2000). Tributyltin: a unique sediment Contaminant. Soil Sediment and Groundwater, June 2000.

University of New South Wales (UNSW) (2002). School of Civil Engineering, Water Research Laboratory, Brooklyn Estuaries Processes Study – Working Paper 3.

USEPA (2003). Ambient aquatic life water quality criteria for tributyltin (TBT) – final. United States U.S. Environmental Protection Agency, Office of Water Report 4304T, EPA 822-R-03-031, Washington, DC, USA, 138 pp.

Voulvoulis, N., Scrimshaw, M. and Lester, J. (2000). Occurrence of four biocides utilized in antifouling paints, as alternatives to organotin compounds, in waters and sediments of a commercial estuary in the UK. Marine Pollution Bulletin, 40, 938-946.

Figure 1A. Sediment Study (Stage 1) Sampling Locations

Figure 1B. Antifoul Study (Stage 2) Sampling Locations

Figure 2. TBT in Sediments

Figure 3. Exceedances of ANZECC/ARMCANZ (2000) Sediment Quality Guidelines

Figure 4. Silt + Clay contents in sediments at sampling locations H1 to H20 in 2007 and previous studies (Sediment Study – Stage 1).

Figure 5. Concentrations of copper in sediments at sampling locations H1 to H20 in 2007 and previous studies (Sediment Study – Stage 1).

Figure 6. Concentrations of copper (normalized to <0.063 mm fraction) in sediments at sampling locations H1 to H20 in 2007 and previous studies (Sediment Study – Stage 1).

Figure 7. Concentrations of TBT and copper in sediments at 16 sampling locations.

Figure 8. Concentrations of TBT in 16 sediment samples from marinas and other sampling locations (Antifoul Study - Stage 2).

Table 1. Descriptive Statistics of Concentrations of Inorganic Analytes and Nutrients - Sediment Study (Stage 1).

Table 2. Concentrations of PAHs (in g/kg) in Whole Sediments - Antifoul Study (Stage 2).

Table 3. Comparison of Sediment Chemistry at 18 Sampling Locations (data from Sediment Study – Stage 1).

Table 4. General Information on Organic Booster Biocides.

Source: Amog Consulting (2002) and references therein.

Table 5. Concentrations of Tributyltin (TBT) and Diuron in Sediments at Sampling Locations H9, H31, H37 and H38.

| Sampling Location | TBT | Diuron |
| --- | --- | --- |
| H9 | 14 | 30 |
| H31* | 3 | <1 |
| H37 | 25 | 9 |
| H38 | 11 | 10 |
| QC5 (H9) | 20 | 40 |
| Concentrations in g/kg |  |  |

Note: TBT-Sn concentrations were converted to TBT (the compound). Molecular weight of TBT = 2.44 x TBT-Sn.